PUBLIC WORKS DEPARTMENT

INTER-OFFICE MEMO

DATE:		December 10, 2008

TO:		Conservation Development and Planning Department

FROM:	Erich Kroll, Supervising Engineer

SUBJECT:	Bennett Lane Winery, 3340 Hwy 128, Calistoga

APN 017-160-002, P07-00299 Mod

The application will allow the applicant to increase the square footage of the production area from 7,552 sf to 15,805 sf.  The applicant is proposing an increase in visitation from 50 visitors in busiest day(see memo dated June 11, 2007 prepared by D. Lander of Public Works) to 125 visitors on the busiest day.  The applicant is not proposing any increase in production or the number of employees.  Parcel is located on SH 128 just north of Tubbs Lane.

EXISTING CONDITIONS:

- Existing access road is 16 feet wide and paved, with 4 feet compacted earthen shoulders on both sides.

- Parking surface is currently paved.

- The average daily traffic count during 2005 for SH 128 was 3850 just north of Tubbs Lane, and was 9200 just south of Tubbs Lane.  These counts are from the CalTrans website.  The estimated annual daily traffic count of 3850 combined with the traffic estimated to be 39-112 trips/day generated by this winery indicate that a left turn lane on SH 128 is required. (County Road and Street Standards, Page 15, Sec. 16 and Page 16-A)

RECOMMENDED CONDITIONS:

##### GROUNDWATER

- We have reviewed the phase one, water availability analysis for the proposed project.  The 10.0 acre parcel is located in the “valley floor” area, with an extraction threshold of            1.0 AF/Acre, resulting in a total parcel threshold of 10.0 AF/Year.  The estimated water demand of 5.05 AF/Year is below the established threshold for the property.  Therefore, the projected water use for this project should not have a significant impact on static water levels of neighboring wells.  No further analysis is necessary.

NEW DRIVEWAY:

- Any improvements to the access drive shall conform to the latest addition of the Napa County Road and Street Standards.  (County Road and Street Standards, Page 9, Par. 12).

PARKING:

- Any change in parking proposed by the applicant or required by the Planning Department as a condition of this use permit modification must have a minimum structural section of 5 inches of Class II Aggregate Base plus a double chip seal coat or equivalent.  (County Road and Street Standards, Page 27, Section 19).

- Parking lot details shall conform to the requirements of the latest edition of the Napa County Road and Street Standards.

SITE IMPROVEMENTS:

- Proposed drainage for the development shall be shown on the improvement plans and shall be accomplished to avoid the diversion or concentration of storm water runoff onto adjacent properties.  Plan shall also indicate the existing drainage path and structures and changes note any changes to the historical runoff.   Drainage calculations shall also include a 100 year analysis of the drainage channel located in the northeast corner of the project.  If the 100 year floodplain is determined within the existing or proposed building footprints, a Floodplain Management Permit will be required prior to issuing Public Works clearance for a Building or Site Grading Permit.

- Grading, drainage and parking improvements shall be constructed according to the latest “Napa County Road and Street Standards”.

- A left hand turn will be required for the project based on current County of Napa Road and Street Standards.  The applicant must obtain an encroachment permit for any work that may be proposed within the Caltrans Right-of-Way.

OTHER RECOMMENDATIONS:

- If any discharge of concentrated surface waters is proposed into any “Waters of the State,” the permittee shall consult with and secure any necessary permits from the State Regional Water Quality Control Board.

- Prior to the issuance of any grading or building permit, or the signing of improvement plans, the permittee and County shall survey and document the condition of County roads before construction begins, and then reevaluate conditions at the end of construction. Prior to Occupancy of any buildings or commencement of any use, the permittee shall be responsible for repair of any pavement degraded due to its construction vehicles.

- All on site civil improvements including but not limited to the excavation, fill, general grading, drainage, curb, gutter, surface drainage, storm drainage, parking, and drive isles, shall be constructed according to plans prepared by a registered civil engineer, which will be reviewed and approved by this office prior to the commencement of any on site land preparation or construction. Plans shall be submitted with the building permit documents at the time of building permit application. A plan check fee will apply.

CONSTRUCTION STORMWATER REQUIREMENTS

- All earth disturbing activities shall include measures to prevent erosion, sediment, and waste materials from leaving the site and entering waterways both during and after construction in conformance with the Napa County Stormwater Ordinance.  Best Management Practices shall also be implemented to minimize dust at all times.

- All hazardous materials stored and used on-site during construction that could cause water pollution (e.g. motor oil, cleaning chemicals, paints, concrete, etc.) shall be stored and used in a manner that will not cause pollution, with secondary containment provided.  Such storage areas shall be regularly cleaned to remove litter and debris.  Any spills shall be promptly cleaned up and appropriate authorities notified.

POST-CONSTRUCTION RUNOFF MANAGEMENT REQUIREMENTS

- Project must conform and incorporate all appropriate site design Best Management Practices as required by the Napa County manual for Post-Construction Runoff Management Requirements which is available at the Public Works office.

- Parking lots and other impervious areas shall be designed to drain through grassy swales, buffer strips, sand filters or other sediment control methods which will be approved by this Department.  If any discharge of concentrated surface waters is proposed into any “Waters of the State,” the permittee shall consult with and secure any necessary permits from the State Regional Water Quality Control Board prior to the issuance of applicable construction permits.

- The application shall incorporate Site Design, Source Control and Treatment control Best Management Practices to comply with County and State water quality standards.

- Prior to final occupancy the property owner must legally record an “implementation and maintenance agreement” approved by the Public Works department to ensure all post-construction structures on the property remain functional and operational for the indefinite duration of the project.

- Each year the entity responsible for maintenance is required to complete an annual report that includes copies of completed inspection and maintenance checklists to document that maintenance activities were conducted during the previous year. The annual report shall be retained for a period of at least five years and made available upon request by the County.

- Loading/unloading dock areas must be covered or designed to preclude stormwater run-on and runoff.  All direct connections to storm drains from depressed loading docks (truck wells) are prohibited.

- Post-development runoff volume shall not exceed pre-development runoff volume for the 2-year, 24-hour storm event. Post-development runoff volume shall be determined by the same method used to determine pre-development conditions.  If post-development runoff volume exceeds pre-development runoff volume after the site design BMPs are incorporated into the project’s overall design, a structural BMP (e.g. bio-retention unit) may be used to capture and infiltrate the excess volume.

- Trash storage areas shall be paved with an impervious surface, designed not to allow run-on from adjoining areas, and screened or walled to prevent off-site transport of trash.  Trash storage areas must contain a roof or awning to minimize direct precipitation or contain attached lids on all trash containers that exclude rain.  All trash enclosures must be covered and protected from rain, roof, and surface drainage.

Any changes in use may necessitate additional conditions for approval.

If you have any questions regarding the above items please contact Erich Kroll at 253-4351.  For groundwater questions, please contact Anna Maria Martinez