PUBLIC WORKS DEPARTMENT

INTER-OFFICE MEMO

DATE:	January 31st, 2008

TO:		Conservation Development and Planning Department

FROM:	Drew Lander, Assistant Engineer

SUBJECT:	Robert Pecota Winery, APN# 017-150-004, File # P07-00472

The application will allow the applicant to establish a winery with a production capacity of 48,000 gal/yr along with a 70,000 lb/yr cheese processing facility and 150 tons/yr of olive oil; 11 full time and 19 part time employees; construct a 20 space winery parking lot and 3 loading areas.  Parcel is located to the east side of Bennett Lane, north of the intersection with Tubbs Lane.

EXISTING CONDITIONS:

There currently is no existing access road.

Parking surface is currently undeveloped.

Future site of all proposed structures is currently unimproved.

Parcel is bordered by the 100-year floodplain boundary of the Napa River, as identified on the Flood Insurance Rate Maps (panel 175A) published by FEMA.

No traffic counts are available for Bennett Lane at the time of this memo.

RECOMMENDED CONDITIONS:

##### GROUNDWATER

- We have reviewed the phase one, water availability analysis for the proposed project.  The 23.96 acre parcel is located in the “valley floor” area, with an extraction threshold of 1.0 AF/Acre, resulting in a total parcel threshold of 23.96 AF/Year.  The estimated water demand of 7.8 AF/Year is below the established threshold for the property.

However, the applicant has submitted a will serve letter from the City of Calistoga for water service for domestic and process purposes, and has provided documentation (Water Rights Permit 16578) that states that 11 acre-feet may be diverted from the Napa River on an annual basis for irrigation purposes.  No on site wells will be used to serve this parcel.

No further analysis is necessary.

NEW DRIVEWAY:

- Access drive shall be a minimum of 18 feet wide with 2 feet of shoulder from the nearest publicly maintained road to the project site. Structural section shall be a minimum 5 inches of Class II Aggregate Base plus a double chip seal coat or equivalent. (County Road and Street Standards, Page 9, Par. 12).

PARKING:

- Any parking proposed by the applicant or required by the Planning Commission as a condition of this use permit must have a minimum structural section of 5 inches of Class II Aggregate Base plus a double chip seal coat or equivalent.  (County Road and Street Standards, Page 27, Section 19).

- Parking lot details shall conform to the requirements of the latest edition of the Napa County Road and Street Standards.

SITE IMPROVEMENTS:

- All earth disturbing activities shall include measures to prevent erosion, sediment, and waste materials from leaving the site and entering waterways both during and after construction in conformance with the Napa County Stormwater Ordinance.  Best Management Practices shall also be implemented to minimize dust at all times.

- Proposed drainage for the development shall be shown on the improvement plans and shall be accomplished to avoid the diversion or concentration of storm water runoff onto adjacent properties.  Plan shall also indicate the path and changes in runoff.

- Grading, drainage and parking improvements shall be constructed according to the latest “Napa County Road and Street Standards”.

- The applicant must obtain an encroachment permit for any work performed within the Napa County Right-of-Way.

OTHER RECOMMENDATIONS:

- At the time of building permit submittal the applicant shall provide all available historic flooding data and associated historic flooding elevations to this department.

- At the time of building permit submittal the applicant shall provide proposed and final elevations of structures and surrounding grades.

- All site improvements must be completed to prevent the displacement of floodwaters onto neighboring properties and shall allow for passing the 100 year storm flow event without an increase in the Base Flood Elevation.

- The application indicates that the construction activity will result in disturbance of greater than one acre of total land area. Therefore, the permittee shall be required to file a Notice of Intent with the California Regional Water Quality Control Board (SRWQCB) prior to any grading or construction activity.  All hazardous materials stored and used on-site that could cause water pollution (e.g. motor oil, cleaning chemicals, paints, concrete, etc.) shall be stored and used in a manner that will not cause pollution, with secondary containment provided.  Such storage areas shall be regularly cleaned to remove litter and debris. Any spills shall be promptly cleaned up and appropriate authorities notified.  Parking lots and other impervious areas shall be designed to drain through grassy swales, buffer strips, sand filters or other sediment control methods which will be approved by this Department.  If any discharge of concentrated surface waters is proposed into any “Waters of the State,” the permittee shall consult with and secure any necessary permits from the State Regional Water Quality Control Board.  All trash enclosures must be covered and protected from rain, roof, and surface drainage.

- Improvements shall be constructed according to plans prepared by a registered civil engineer, which will be reviewed and approved by this office.  Improvement plans shall be submitted with the building permit.  A plan check fee will apply.

- Prior to the issuance of any grading or building permit, or the signing of improvement plans, the permittee and County shall survey and document the condition of County roads before construction begins, and then reevaluate conditions at the end of construction. Prior to Occupancy of any buildings or commencement of any use, the permittee shall be responsible for repair of any pavement degraded due to its construction vehicles.

Any changes in use may necessitate additional conditions for approval.

If you have any questions regarding the above items please contact  Drew Lander, or Jeannette Doss at 253-4351.  For groundwater questions, please contact Anna Maria Martinez.