Napa County Defensible Space Guidelines

A.  Purpose

These guidelines are intended to provide property owners with examples of fuel modification measures that should be used to create defensible space around structures.  A defensible space perimeter provides firefighters with a working environment that allows them to protect buildings and structures from encroaching wildfires and minimizes the chance that a structure fire will escape to the surrounding wildland.

These guidelines apply to any person who owns, leases, controls, operates, or maintains a building or structure in, upon, or adjoining any mountainous area, forest-covered lands, brush-covered lands, grass-covered lands, or any land that is covered with flammable material, and located within Napa County.

The vegetation surrounding a building or structure is fuel for a fire. Even the building or structure itself is considered fuel. Research and experience have shown that fuel reduction around a building or structure increases the probability of it surviving a wildfire. Good defensible space allows firefighters to protect and save buildings or structures safely without facing unacceptable risk to their lives. Fuel reduction through vegetation management is the key to creating good defensible space.

Here are some common practices that guide fuel modification treatments to ensure creation of adequate defensible space:

• Creation of defensible space through vegetation management usually means reducing the amount of fuel around the building or structure, providing separation between fuels, and/or reshaping retained fuels by trimming.  Defensible space can be created by removing dead vegetation, separating fuels, and pruning lower limbs.

• In all cases, fuel reduction means arranging the tree, shrubs and other fuel sources in a way that makes it difficult for fire to transfer from one fuel source to another. It does not mean cutting down all trees and shrubs, or creating a bare ring of earth across the property.

• A homeowner’s clearing responsibility is limited to 100 feet away from all structures on the homeowner’s property and adjacent properties.  Homeowners are required to clear 100 feet from structures or to the property line, whichever is less.  If fuel conditions on Property A create a fire hazard on an adjacent parcel because that fuel is within 100 feet of a structure, then the owner of Property A where the fuel hazard exists is responsible for clearing the required defensible space.

• Homeowners who do fuel reduction activities that remove or dispose of vegetation are required to comply with all federal, state or local environmental protection laws and obtain permits when necessary. For example, trees removed for fuel reduction that are used for commercial purposes require permits from the California Department of Forestry and Fire Protection (CAL FIRE).  Environmental protection laws include, but are not limited to, those dealing with threatened and endangered species, water quality, air quality, and cultural/archeological resources. Contact your local resource or planning agency officials to ensure compliance.

The methods used to manage fuel can be important in the safe creation of defensible space. Care should be taken with the use of equipment when creating your defensible space zone. Internal combustion engines must have approved spark arresters; metal cutting blades (on lawn mowers or weed trimmers) should be used with caution during periods of high fire danger to prevent starting fires. A metal blade striking a rock can create a spark and start a fire, a common cause of fires during summertime.

Vegetation removal can also cause soil disturbance, soil erosion, and regrowth of vegetation, and introduce non-native invasive plants. Always keep soil disturbance to a minimum, especially on steep slopes. Erosion control techniques such as minimizing use of heavy equipment, avoiding stream or gully crossings, using mobile equipment during dry conditions, and covering exposed disturbed soil areas will help reduce soil erosion and plant regrowth.

Areas near water (riparian areas), such as streams or ponds, are a particular concern for protection of water quality. To help protect water quality in riparian areas, avoid removing vegetation associated with water, avoid using heavy equipment, and do not clear vegetation to bare mineral soil.

B. Definitions

Defensible Space: The area within a parcel where basic wildfire protection practices are implemented, providing the key point of defense from an approaching wildfire or escaping structure fire. The area is characterized by the establishment and maintenance of emergency vehicle access, emergency water reserves, street names and building identification, and fuel modification measures.

Aerial fuels: All live and dead vegetation in the forest canopy or above surface fuels, including tree branches, twigs and cones, snags, moss, and high brush. Examples include trees and large bushes.

Building or structure: Any structure used for support or shelter of any use or occupancy.

Flammable and combustible vegetation: Fuel as defined in these guidelines.

Fuel: Vegetative material, live or dead, which is combustible during normal summer weather.

Homeowner: Any person who owns property or maintains a building or structure in, upon, or adjoining any mountainous area, forest-covered lands, brush-covered lands, grass-covered lands, or any land that is covered with flammable material, and located within Napa County.

Ladder Fuels: Fuels that can carry a fire vertically between or within a fuel type.

Reduced Fuel Zone: The area between 30 and 100 feet away from the building or structure.

Surface fuels: Loose surface litter on the soil surface, normally consisting of fallen leaves or needles, twigs, bark, cones, and small branches that have not yet decayed enough to lose their identity; also grasses, forbs, low and medium shrubs, tree seedlings, heavier branches and downed logs.

C. Fuel Treatment Guidelines

The following fuel treatment guidelines comply with the requirements of 14 California Code of Regulations 1299, California Public Resources Code section 4291, and the Napa County Fire Protection Weed Abatement Ordinance.

All persons using these guidelines to comply with CCR 1299, PRC 4291 and the Napa County Fire Protection Weed Abatement Ordinance shall implement General Guidelines 1., 2., 3., and either 4a or 4b., as described below.

General Guidelines:

1. Maintain a firebreak by removing and clearing away all flammable vegetation and other combustible growth within 30 feet of each building or structure. Single specimens of trees or other vegetation may be retained provided they are well-spaced, well-pruned, and create a condition that avoids spread of fire to other vegetation or to a building or structure.

2. Dead and dying woody surface fuels and aerial fuels within the Reduced Fuel Zone shall be removed. Loose surface litter, normally consisting of fallen leaves or needles, twigs, bark, cones, and small branches, shall be permitted to a depth of 3 inches. This guideline is primarily intended to eliminate trees, bushes, shrubs and surface debris that are completely dead or with substantial amounts of dead branches or leaves/needles that would readily burn.

3. Down logs or stumps anywhere within 100 feet from the building or structure, when embedded in the soil, may be retained when isolated from other vegetation. Occasional (approximately one per acre) standing dead trees (snags) that are well-spaced from other vegetation and which will not fall on buildings or structures or on roadways/driveways may be retained.

4. Within the Reduced Fuel Zone, one of the following fuel treatments (4a. or 4b.) shall be implemented. Properties with greater fire hazards will require greater clearing treatments. Combinations of the methods may be acceptable as long as the intent of these guidelines is met.

4a. Reduced Fuel Zone: Fuel Separation

In conjunction with General Guidelines 1., 2., and 3., above, minimum clearance between fuels surrounding each building or structure will range from 4 feet to 40 feet in all directions, both horizontally and vertically.

Clearance distances between vegetation will depend on the slope, vegetation size, vegetation type (brush, grass, trees), and other fuel characteristics (fuel compaction, chemical content, etc.). Properties with greater fire hazards will require greater separation between fuels. For example, properties on steep slopes having large sized vegetation will require greater spacing between individual trees and bushes (see Plant Spacing Guidelines below).  Groups of vegetation (numerous plants growing together less than 10 feet in total foliage width) may be treated as a single plant. For example, three individual manzanita plants growing together with a total foliage width of eight feet can be “grouped” and considered as one plant and spaced according to the Plant Spacing Guidelines in this document.

Clearance requirements include:

• Horizontal clearance between aerial fuels, such as the outside edge of the tree crowns or high brush.  Horizontal clearance helps stop the spread of fire from one fuel to the next.

4 ft. to 40 ft

depending on slope and vegetation type and size

Trees							Shrubs

10 ft. to 30 ft.

depending on slope and vegetation type and size

Horizontal clearance between aerial fuels

• Vertical clearance between lower limbs of aerial fuels and the nearest surface fuels and grass/weeds.  Vertical clearance removes ladder fuels and helps prevent a fire from moving from the shorter fuels to the taller fuels.

4 ft to 40 ft. depending on slope and vegetation size/type

Vertical clearance between aerial fuels

| Plant Spacing Guidelines Guidelines are designed to break the continuity of fuels and be used as a “rule of thumb” for achieving compliance with the Napa County Weed Abatement Ordinance. |  |  |  |
| --- | --- | --- | --- |
| TREES | Minimum horizontal space from edge of one tree canopy to the edge of the next |  |  |
|  | Slope | Spacing |  |
|  | 0% to 20% | 10 feet |  |
|  | 20% to 40% | 20 feet |  |
|  | Greater than 40% | 30 feet |  |
| SHRUBS | Minimum horizontal space between edges of shrubs |  |  |
|  | Slope | Spacing |  |
|  | 0% to 20% | 2 times the height of the shrub |  |
|  | 20% to 40% | 4 times the height of the shrub |  |
|  | Greater than 40% | 6 times the height of the shrub |  |
| VERTICAL SPACE | Minimum vertical space between top of shrub and bottom of lower tree branches: 3 times the height of the shrub |  |  |

4b. Reduced Fuel Zone: Defensible Space with Continuous Tree Canopy

To achieve defensible space while retaining a stand of larger trees with a continuous tree canopy apply the following treatments:

• Generally, remove all surface fuels greater than 4 inches in height. Single specimens of trees or other vegetation may be retained provided they are well-spaced, well-pruned, and create a condition that avoids spread of fire to other vegetation or to a building or structure.

• Remove lower limbs of trees (“prune”) to at least 6 feet up to 15 feet (or the lower 1/3 branches for small trees). Properties with greater fire hazards, such as steeper slopes or more severe fire danger, will require pruning heights in the upper end of this range.